# Geographic Information Systems

# Project Report

Proposed by :

Aziza Ben Tanfous :   n° 27156

Johanna Rauberger  :   n° 27492

Proposed to :

Professor Rui Figueira

# Problem 01 – Regions vulnerable to flood risk

In this exercise, we used QGIS to determine the regions vulnerable to flood risk due to river overflow during the raining season. The study area extent was defined by specific coordinate values.

Data Acquisition: We downloaded the SRTM DEM data from the provided URL and loaded it into QGIS. We also loaded the Parcels2023.gpkg.

Processing Steps: The procedure involved using the Raster calculator to generate a binary layer that represents the extent of river overflow. This binary layer was then converted to a polygon vector layer and clipped to the study area extent. After filtering for polygons with a height of 17.0m or less, the area of the vulnerable regions was computed and stored in the attribute table of the vector layer. Furthermore, a new vector layer was created to categorize the vulnerable regions into high risk, moderate risk, and low risk based on their heights. Subsequently, the area of the vulnerable regions in each risk category was calculated and stored in the attribute table of the vector layer.

Results: Within the given study area, about 235 km² are vulnerable to flood risk due to river overflow. Out of that area, 16704 ha classified as high risk (elevation <10m), 4751 ha as moderate risk (10m ≤ elevation <15m) and 2012 ha as low risk (15m ≤ elevation ≤ 17m), see Fig1.

Fig1: Regions vulnerable to flood risk due to river overflow classified by risk level.

# Problem 02 – Soil use

For this problem, our assigned area was Region 4 from the EditingZones layer, which represents a specific polygon. Our objective was to identify and delineate parcels within Region 4 using Google Satellite imagery. Through careful analysis of land use patterns, boundaries, and visible features, we aimed to accurately represent the soil use within the area. To store the vector data and attribute information, we created a new GeoPackage file named Probl2_27156.gpkg as per the assignment requirements. We ensured consistent spatial reference by setting the GeoPackage file to use the ETRS-TM06 CRS (EPSG: 3763). Within the GeoPackage file, we established a dedicated vector layer named Parcels4 to represent the parcels in Region 4. Digitizing approximately 10 parcels based on our interpretation of the satellite imagery, we assigned the corresponding soil use codes and descriptions by referring to the provided "TableSoilUse.xls" file using the joint operation. Additionally, we assigned owners to each parcel, obtained from the "Owners.xls" file using the joint operation. All vector layers, including the Parcels4 layer, were stored within the GeoPackage file to ensure complete coverage of the assigned area in Region 4.

# Problem 03 - all parcels in the region delimited by Cadastre and those created in Problem 02

Data Preparation: We gathered the necessary input datasets, including the Cadastre GDS, Parcels_27156 from Problem 02, and SoilUse GDS. Additionally, modify the TableSoilUse.xls to include a new class UseCode: 50  with the description Other.

Processing Steps: We began by opening the required datasets in QGIS, including Cadastre, Parcels_27156, SoilUse, and TableSoilUse.xls. We created a new Geopackage layer named ParcelsNew_27156 and saved it in the DataOut folder. The attribute table of ParcelsNew_27156 was updated to include fields for soil use, Parcels_27156, and owner. A legend based on soil use was created using the "Categorized" symbolization method. We used the field calculator to compute the area of each parcel and converted it to hectares. The area owned by each owner and the total area for each soil use class were calculated using the groupstat function and saved in a CSV files. Lastly, we used the Extract by Attribute tool to create a new layer named Use_27156 that represents only agriculture and forest regions from the ParcelsNew_27156 dataset, excluding other classifications.

# Problem 04 - the relative index of susceptibility to groundwater pollution of the region

Data Preparation: We gathered the necessary input datasets, including the ParcelsNew_27156 and the SoilType GDS.

Processing Steps: The analysis commenced by accessing the necessary datasets, including ParcelsNew_27156 and SoilType GDS. Subsequently, a new attribute, namely 𝐼𝐼𝑢  was introduced to the ParcelsNew_27156 layer utilizing the field calculator. Similarly, the SoilType GDS was augmented with a new attribute called 𝐼𝐼𝑢𝑢 through the same procedure. Through a data join operation, the information from the SoilType GDS was merged with the ParcelsNew_27156  layer, resulting in the creation of the GwPollIndex  layer. The calculation of the 𝐼𝐼𝑝 value was carried out by employing the formula 𝐼𝐼𝑝𝑝 = 0.25𝐼𝐼𝑢𝑢 × 𝐼𝐼 . To visualize the GwPollIndex layer, symbology techniques were employed, utilizing the classification provided in the assignment document. Subsequently, the select by expression functionality was utilized to identify and isolate attributes where 𝐼𝐼𝑝𝑝 exceeded 3. Finally, the selected features were exported as a new layer denoted as HvHGwPollIndex.

Problem 05 – Highest and lowest producers of olive oil in the world

Question: What countries produce olive oil, and which country had the highest and lowest production of olive oil in 2020?

Data Acquisition: We downloaded the data from the FAOSTAT database and the world map from open data soft.

Data Management:

In order to ensure the quality and reliability of the data, we conducted a thorough data cleaning process. The steps involved removing duplicate entries with df.drop_duplicates(), handling missing values by replacing 'NA' with NaN using df.replace('NA', pd.NA), dropping rows with missing values using df.dropna(). The resulting clean data was then displayed using print(df.head()), demonstrating the removal of duplicates, handling of missing values, and any necessary data type corrections. These measures contributed to improved data integrity and enhanced the suitability of the dataset for subsequent analysis.

We removed also certain columns that were deemed unnecessary or redundant. These columns included Flag, Flag Description, Area Code (M49), Domain Code, Domain, Element Code, Element, Item Code (CPC), and Year Code. Removing these columns helped streamline the dataset and focus on the relevant information for analysis.  Finally we downloaded the modified_table.csv  ( Fig 2) . The modified_table has been meticulously normalized to meet the requirements of the 3rd normal form, guaranteeing data consistency, eliminating redundancy, and streamlining data management.

Fig 2: Cleaning Code using python

Processing Steps:

In the GIS analysis workflow, we commenced by opening the dataset comprising the olive oil production data and the world map in QGIS. After confirming that both datasets share the same Coordinate Reference System (CRS) of WGS 84_EPSG:4326, we proceeded to create a new layer named "OliveOil_producers" by performing a join operation between the olive oil production data and the spatial data layer containing the country boundaries. To streamline the layer, we selectively retained only the name and continent attributes from the world map layer, discarding unnecessary features. Next, utilizing the Rule-Based symbology option in QGIS, we developed a thematic map based on the olive oil production values. The color blue was assigned to represent the olive oil producers on the map, ensuring clarity and visual distinction. Additionally, to standardize the production values, we introduced a new feature named "production" that ensured all values were expressed in decimal format with three digits after the decimal point. To identify the highest and lowest producers, we employed the "select by expression" functionality, enabling us to select the relevant features based on their production values. Subsequently, we extracted the corresponding layer containing the selected features for further analysis. In order to present our findings effectively, we utilized the new print layout feature in QGIS to create a well-composed map. The map layout incorporated essential elements such as a title, scale bar, north arrow, and a legend, providing a comprehensive and visually appealing representation of the olive oil production analysis. By following this professional workflow, we were able to analyze and visualize the olive oil production data, facilitating a clear understanding of the highest producers and conveying the information in an organized and visually engaging manner.

Results:

The production of olive oil is a significant industry worldwide, involving several countries renowned for their unique flavors and characteristics in olive oil products. Spain, Italy, Greece, Turkey, Tunisia, Morocco, Portugal, and United States are among the major contributors to the global olive oil market.Spain led global olive oil production in 2020 with 1,356,411,000 tonnes. In contrast, Ukraine faced challenges due to its continental climate and agricultural focus, resulting in negligible olive oil production of 0 tonnes. The country's limited demand for olive oil further hindered investment in its cultivation and production (Fig3).

Fig 3: Production of olive oil

The study area extent given in ETRS PT-TM06 coordinate values were converted into EPSG:4326 WGS 84 format to comply with input requirements to download the SRTM DEM data.